OWNER’S MANUAL

2018/2019

MODEL YEARS

Contender Boats, Inc.

1820 SE 38th Avenue

Homestead, FL 33035

(305) 230-1600

# Table of Contents

Table of Contents	2

Welcome	3

Background	4

Warranty Information	6

Safety Information	9

Battery Switch Operation	10

Fuel System	11

Standard Fuel Capacities	12

Head System	13

Livewell System	13

Designated Occupant Positions	14

Maximum Outboard Horsepower Ratings	15

Gelcoat Maintenance	16

Upholstery Care	18

Reboarding Instructions	18

Boat/Engine Information	19

Customer Satisfaction Survey	20

# Welcome

Congratulations on the purchase of your new Contender.

Contender Boats is happy to provide you with this Owner’s Manual as a way to help you become familiar with the many systems aboard your boat.  The Manual is not intended to be a full explanation of how to maintain and operate your Contender, but is instead a supplement to help you along the way.

Also please note that due to the custom nature of all Contender Boats, the information in this manual may vary from what is installed on your boat.

If you have any further questions feel free to contact your local dealer or Contender directly and a member of our staff will be happy to help.

# Background

Introduction

If there’s one thing we’ve proven, it’s this: you don’t have to be around a long time to be considered a legend. In just 30 years, Contender has redefined all that a sportfishing boat can be.

From our early beginnings, we’ve evolved from the original hull design of the classic 25’ to the sophisticated power and performance of today’s tournament proven winners. And unlike some mass production boat builders, we at Contender pride ourselves with a traditional “hands on” boat building philosophy while using the most advanced modern materials and quality components available. But just like the people we aim to please, we too are never satisfied. That is why we will continue to do all we can to refine and improve the concept of the ultimate sportfishing boat.

Why Contender?

You know there is more to it than just luck. There's fishing the right bait, the water temperature, the wind, the season and a host of other factors that stand between you and the scale at the dock. And then there's the boat...

We'll put it simply: the boat matters. To own a Contender is to own the best sport fishing boat on the market. Period.

Since 1984, Contender has been producing the highest quality, most well designed boats. Our popularity among true die-hard fisherman is a testament to Contender's reputation and legendary performance.

However, even the best designed boat, with the highest quality construction materials means absolutely nothing if the hull design is not proven. Every Contender is crafted using our proven deep-V 24.5° deadrise hull that incorporates our reverse radius chine for a dry ride in the roughest conditions. Not only does this create added lift on plane to run faster on less fuel, it also provides a rock steady fishing platform at rest.

A Contender is built by fishermen for fishermen. That means we know how fishermen think. That’s why our interior layouts are clean and unobstructed with one level deck allowing quick and easy movement from bow to stern. Below our oversized decks lie large storage compartments with waterproof gaskets. We know that fishermen are very particular about their boats. As a semi-custom manufacturer, Contender, through your dealer, can make modifications to meet your personal fishing style. Making these boats not just great for fishing, but any boating activity.

Other noteworthy Contender features include raised transom live wells, integrated flush mount bow rails, anchor lockers, walk-through transoms and self-bailing cockpits. And what good would the best fishing boat be without the best outboards to power it? Every Contender is outfitted with Yamaha outboards that are perfectly suited for the Contender model you choose.

So whether you're partial to the canyons, the Outer Banks or the Gulf, we wish you luck. The fish and Mother Nature aren't going to make it easy, that's why you buy a Contender.

Company History

Contender Boats was founded in 1982 by its current owner, Joseph Neber. Incorporated in the state of Florida in 1984, it has operated continuously since then producing over 6000 boats in sizes from 21’ (6.5m) to 40’ (12.2m) in open and cabin configurations. Contender produces high quality outboard powered offshore performance cruising, luxury, and fishing vessels.

Our Facility

The Contender production facility is located in Homestead, Florida.

Dedication to Quality

We don’t build decent boats.

Decent boats don’t win tournaments. Serious fishermen don’t buy decent boats. But most importantly, decent boats don’t inspire the kind of loyalty that Contender owners display; a loyalty that comes from having 110% confidence in the design and construction of your vessel.

At Contender, we are singularly committed to producing the best boat in the world. That’s why every Contender is crafted with hand laminated solid fiberglass using biaxial and triaxial cloths, structural PVC core sandwich construction for lightweight and superior strength as well as vinyl ester resins and specialty formulated gel coats for high gloss mirror-like finishes. We want our boats to perform and look as good as you do with that trophy in your hand.

# Warranty Information

# Safety Information

Safe boating is very important to Contender Boats. In addition to the information found in this manual, it is recommended that any boat owner or operator familiarize themselves with state/local laws as well as all U.S. Coast Guard safety guidelines. It is also important to have a good understanding of the Rules of the Road and to exercise good judgment and common sense at all times while on the water.

Contender also advises that while on board, all passengers should wear lifejackets, and the vessel’s operator should always wear the engine safety lanyard.

You should never operate the vessel under the influence of drugs or alcohol. Additionally, extreme caution should be exercised while operating the vessel in rough water, severe weather, darkness, fog or low visibility.

As per ABYC T-5, the following warning labels are placed on each Contender Boat (where applicable). If any of the below labels is missing, becomes damaged or becomes unreadable, please contact Contender or your dealer for a replacement.

- MAXIMUM CAPACITIES (for boats under 26’)

- CARBON MONOXIDE WARNING – at helm

- CARBON MONOXIDE WARNING – at transom

- ROTATING PROPELLER WARNING – at helm

- ROTATING PROPELLER WARNING – at transom

- FUEL SYSTEM WARNING – in bilge

- FUEL SYSTEM PRESSURE WARNING – at fuel fill

- TRASH OVERBOARD REGULATIONS (for boats 26’ and over)

- OIL DISCHARGE PROHIBITED (for boats 26’ and over)

# Battery Switch Operation

# Fuel System

Many Contenders come equipped with several fuel tanks.  Most often, the configuration is a center tank and two saddle tanks.

The following diagram is provided to help understand how the valves (located in the bilge compartment) control fuel flow to each of the boat’s motors

When equipped with three fuel tanks, there will be one fill located on the starboard side (for the starboard saddle tank) and two located on the port side. One of the fills on the port side will have a small “C” etched in it, to signify the center tank.

All new Contenders come equipped with a Blu Skies pressure-relief EPA compliant fuel system. The fuel fill also includes a pressure-relieving vent, which releases vapors during the diurnal venting cycle. This means that you may notice a small amount of pressure in the system when opening the fuel cap.

On the following page is a list of standard fuel tank sizes for all current Contender Boats models. It is important to note that in many instances, we build boats with custom-sized fuel tanks. If you are unsure of the fuel capacity of your boat, please contact your local dealer or Contender directly.

# Standard Fuel Capacities

22 SPORT			100 GALLONS (CENTER)

24 SPORT			130 GALLONS (CENTER)

25 BAY			90 GALLONS (CENTER)

25 TOURNAMENT		175 GALLONS (CENTER)

28 SPORT			185 GALLONS (CENTER)

28 TOURNAMENT		185 GALLONS (CENTER)

30 TOURNAMENT/ST	115 GALLONS (CENTER)

75 GALLONS (SADDLES)

32 TOURNAMENT/ST	130 GALLONS (CENTER)

90 GALLONS (SADDLES)

35 TOURNAMENT/ST	150 GALLONS (CENTER)

125 GALLONS (SADDLES)

39 TOURNAMENT/ST	230 GALLONS (CENTER)

135 GALLONS (SADDLES)

40 EXPRESS			220 GALLONS (CENTER)

120 GALLONS (SADDLES)

Please note that due to the various components installed in the fuel system to comply with the EPA Evaporative Emissions Regulations, the actual usable volume of each tank may be less than the values listed above.

All fuel systems have been factory inspected and pressure tested in accordance with regulations in effect at the time of manufacture. This inspection ensures that the system is air tight, leak proof and safe. It is the responsibility of the end user to maintain it in that condition. Make frequent inspections of the entire system to ensure it remains functional for years to come.

# Head System

When equipped with a marine toilet, your Contender will include a Y-valve and a holding tank. The Y-valve is typically located on the forward bulkhead of the console interior. When the valve handle is pointed to port, the discharge from the toilet is diverted into the holding tank. When the handle is pointed to starboard, the discharge is flushed overboard. An easy way to remember this is when the vessel is in port (near shore), federal law requires waste to be collected in the holding tank: in port, handle to port.

# Livewell System

Contenders can be built with a wide array of livewell quantities and arrangements. The plumbing of these wells is often specific only to the particular boat or configuration, however, some similarities exist.

Most wells utilize an intake, a drain, and an overflow. The intake is usually from a “high speed water pickup” located on the bottom of the hull. This intake allows the system to receive clean water even while up on plane. This intake is connected to a valve and then a livewell pump. The drain and the overflow are plumbed out the transom, using a seacock valve and a thru-hull fitting.

In most cases, when using the livewell, the drain must be closed to allow water to accumulate in the well. The valve for the overflow can also be closed slightly to allow the flow from the intake to outpace the overflow. This will create a “pressurized” well where the water will fill up to the livewell lid. This helps to reduce/eliminate sloshing inside of the well and will improve bait health.

# Carbon Monoxide Detection

If Carbon monoxide detector sounds it is best to get to fresh air outside the cabin or console immediately. Shut down generator if it is running and ventilate all enclosed areas.

# Designated Occupant Positions

As per ABYC H-5.5.2.2, all vessels under 26’ which come equipped with a maximum capacity sticker must also identify “designated occupant positions” for that number of passengers.  This is to be used as a guideline for passenger placement while underway.

# Maximum Outboard Horsepower Ratings

As per ABYC H-26.5.1.3, all vessels over 26’ shall have a maximum power capacity rating, to be displayed in the manufacturer’s owner’s manual. Please note that these values are subject to change and it is advised to contact Contender before performing a repower.

28 T			600 HP

28 SPORT	600 HP

30 T			600 HP

30 ST		600 HP

32 T			700 HP

32 ST		700 HP

35 T			1050 HP

35 ST		1050 HP

39 T			1400 HP

39 ST		1400 HP

39 FA		1400 HP

40 EX		1100 HP

# Gelcoat Maintenance

# Upholstery Care

The vinyl upholstery used on the seats, cushions, bolsters, and for the headliner in some cabins, should be cleaned periodically with mild soap and water.

Any stain, spill or soiling should be cleaned up promptly to prevent the possibility of permanent staining. When cleaning, always rub gently. Avoid using products containing ammonia, powdered abrasive cleaners, steel wool, ink, strong solvents, acetone and lacquer solvents or other harsh chemicals as they can cause permanent damage or shorten the life of vinyl. Never use steam heat, heat guns or hair dryers on vinyl.

Stronger cleaners, detergents and solvents may be effective in stain removal, but can cause either immediate damage or slow deterioration. Lotions, sunscreen, waxes and polishes, etc., contain oils and dyes that can cause stiffening and staining of vinyls.

# Winterization Procedures

For winterization of your vessel all waste and water tanks should be drained completely and filled with red or pink antifreeze. We recommend red or pink antifreeze so that it can be easily in clear water lines.

Raw water systems should be closed at the sea cock and all water removed from the system. The bilge should be thoroughly cleaned and dry as best as possible.

If your vessel is equipped with a refrigerator or freezer in needs to be turned off and cleaned throughout. Once clean the doors should be propped open for ventilation.

Vessels equipped with air conditioning need to have the heat exchanged flushed of all water and replaced with antifreeze to prevent water freezing and expanding.

# Reboarding Instructions

As per ABYC H-41.10.1.1, each boat model must have a written procedure describing the means of unassisted reboarding. So while most Contender Boats are equipped with boarding ladders, in the event they are not, the following explains how to re-enter the boat should you find yourself overboard with no readily available assistance.

22 Sport/24 Sport

Like most models, the starboard side trim tab can be temporarily used as a step in an emergency. The person reboarding the boat can then step on the engine anti-ventilation plate with their other foot and then use the stern cleat to pull himself or herself up on to the swim platform.

25 Bay

Either the port or starboard trim tab can be used as a step, again in a temporary, emergency situation. The person reboarding the vessel can then use the gunwale lip surrounding the aft casting deck to pull themselves up where they can put their knee on the jackplate and hoist themselves into the boat.

25T, 28S, 28T, 30T/ST, 32T/ST, 35T/ST, 39T/ST/FA, 40 EX

Like the 22/24 Sport, the starboard side trim tab can be temporarily used as a step in an emergency. The person reboarding the boat can then step on the nearest engine anti-ventilation plate with their other foot and then use the stern cleat to pull himself or herself up on to the swim platform.

# Boat/Engine Information

For your convenience, record the information on your boat here:

Model:			____________________________________________

Hull Identification #:	____________________________________________

Date Purchased:		____________________________________________

Dealer:			____________________________________________

Engine Make/Model:	____________________________________________

Engine Serial # (1):	____________________________________________

Engine Serial # (2):	____________________________________________

Engine Serial # (3):	____________________________________________

# Customer Satisfaction Survey